you have hourly NASA/POWER meteorological and solar radiation data (2018–2025) for Lahore. This dataset is rich and can be used for solar energy, climate, or environmental modeling. Let’s go step by step.

1. Data Overview

Your columns are:

| Category | Variables | Description |
| --- | --- | --- |
| Time | YEAR, MO, DY, HR | Temporal resolution — hourly |
| Solar Radiation (Wh/m²) | CLRSKY_SFC_SW_DWN, ALLSKY_SFC_SW_DWN, ALLSKY_SFC_SW_DNI, ALLSKY_SFC_SW_DIFF | Different measures of solar irradiance (clear-sky vs. all-sky, direct vs. diffuse) |
| Geometry | SZA | Solar zenith angle — position of sun |
| Meteorology | T2M (°C), QV2M (g/kg), RH2M (%), PS (kPa), WS10M (m/s), WD10M (°) | Atmospheric conditions |
| Metadata | Elevation, coordinates, etc. | Already in header |

2. Possible Output Variables (Targets)

Depending on what you want to predict or analyze, you can define output variables (dependent variables).
Here are some common and meaningful options:

| Objective | Output Variable | Explanation |
| --- | --- | --- |
| 🌞 Solar Power / Irradiance Modeling | ALLSKY_SFC_SW_DWN | Predict total solar energy available at the surface. |
| ☀️ Direct Solar Radiation Prediction | ALLSKY_SFC_SW_DNI | Useful for concentrated solar systems. |
| ☁️ Diffuse Radiation Estimation | ALLSKY_SFC_SW_DIFF | Important for PV systems under cloud cover. |
| 🌡️ Temperature Forecasting | T2M | Predict air temperature using previous weather and radiation data. |
| 💨 Wind Energy / Dynamics | WS10M | Predict wind speed at 10 m for wind energy modeling. |
| 💧 Humidity / Heat Index Modeling | RH2M or QV2M | For thermal comfort or evapotranspiration estimation. |
| 🔆 Solar Energy Potential | Derived from ALLSKY_SFC_SW_DWN (daily or monthly sum) | Use for PV yield or energy production modeling. |

👉 For solar energy applications, the most common output variable is:

ALLSKY_SFC_SW_DWN (All-sky surface shortwave downward irradiance)

Recommended Variable Name Mapping

| Original Name | New Readable Name | Description |
| --- | --- | --- |
| CLRSKY_SFC_SW_DWN | ClearSkyRadiation | Clear-sky surface shortwave downward irradiance (Wh/m²) |
| ALLSKY_SFC_SW_DWN | SolarRadiation | All-sky surface shortwave downward irradiance (Wh/m²) |
| ALLSKY_SFC_SW_DNI | DirectRadiation | All-sky direct normal irradiance (Wh/m²) |
| ALLSKY_SFC_SW_DIFF | DiffuseRadiation | All-sky diffuse irradiance (Wh/m²) |
| SZA | SolarZenith | Solar zenith angle (degrees) |
| T2M | Temperature | Air temperature at 2 m (°C) |
| QV2M | HumiditySpecific | Specific humidity at 2 m (g/kg) |
| RH2M | HumidityRelative | Relative humidity at 2 m (%) |
| PS | Pressure | Surface pressure (kPa) |
| WS10M | WindSpeed | Wind speed at 10 m (m/s) |
| WD10M | WindDirection | Wind direction at 10 m (°) |

To fill the missing values from the 7th to the 10th month of 2025 year:

Best Option for You: Multi-Year Climatological Replacement

Since you already have 2018–2024 data,
you can use those years to compute a climatological mean (average pattern) for each variable —
for the same month and hour of the day.

That means:

“At 10 AM in July, what’s the average temperature/radiation across 2018–2024?”

Then, fill the 2025-07 → 2025-10 period with those corresponding averages.

Why This Is the Best Option

✔ Keeps correct hourly patterns (day–night, solar rise/set)
✔ Keeps correct seasonal cycle (summer vs fall)
✔ Avoids external bias (since same location + same data source)
✔ Ensures smooth continuity for your models

Augmented Dickey–Fuller (ADF) Test Interpretation

| Metric | Meaning | Your Result |
| --- | --- | --- |
| ADF Statistic | Measures how strongly the series deviates from non-stationarity. The more negative, the stronger the evidence of stationarity. | -14.31 → Very strong stationarity |
| p-value | Probability that the data is non-stationary. If p < 0.05, you can reject the null hypothesis of non-stationarity. | 1.2 × 10⁻²⁶ → Practically zero |

Conclusion

Your SolarRadiation time series is stationary.
That means:

- No significant long-term upward or downward trend.

- Mean and variance are stable over time.

- Perfectly suitable for time-series models (like ARIMA, SARIMAX, Prophet, or LSTM) without differencing.

Problem Context

Your dataset is hourly NASA POWER data (2018–2025) for Lahore, including:

- Solar irradiance variables (ALLSKY_SFC_SW_DWN, etc.)

- Meteorological variables (temperature, humidity, wind speed, etc.)

You correctly observed:

For about 12 hours daily (nighttime), solar irradiance = 0.

That’s expected — the sun simply isn’t up.

So what’s the issue?

If you model or analyze all 24 hours, you include:

- Half the data (nighttime) that is always zero or near-zero for solar radiation.

- This introduces:

  - Strong daily periodicity (false autocorrelation)

  - Non-Gaussian noise (lots of zeros)

  - Distorted correlation results (e.g., solar radiation vs. temperature)

  - Misleading model training for predictive or ML models (SARIMA, LSTM, etc.)

In short — your model wastes effort learning that night is dark.

What Studies & Research Say

Scientific literature and solar energy modeling practices agree on this:

| Source | Practice |
| --- | --- |
| National Renewable Energy Laboratory (NREL) | Solar irradiance modeling usually limited to daylight hours (solar elevation > 0°). |
| Liu & Jordan (1960s) — classic irradiance model | Only daylight considered for daily solar radiation estimates. |
| PV and solar forecasting studies (e.g., Sharma et al. 2021, Solar Energy journal) | Commonly remove nighttime data (irradiance = 0) for regression or forecasting models. |
| MERRA-2 / NASA POWER research papers | Use 06:00–18:00 local time (≈sunrise–sunset window) for surface irradiance modeling. |

Conclusion

| Use Case | Time Window | Rationale |
| --- | --- | --- |
| Solar radiation forecasting (your case) | Daylight only (SolarZenith < 90°) | Avoids constant-zero data, improves accuracy |
| Daily energy estimates | 24-hour | Summing irradiance over full day |
| Seasonal/climate comparisons | 24-hour | Keeps full diurnal context |